## Appendix A: The 32 Construction Data Formats — Quick Reference

This appendix provides a condensed reference for the 32 principal data formats encountered across the construction project lifecycle, ranked from most critical for AI/data integration (#1) to most niche (#32). For each format, the table describes its structure type, human and machine readability, typical generation context, and industry prevalence. Three principles should guide students working with these formats: (1) format does not equal quality — a well-structured IFC file with empty property sets is less useful than a messy CSV with complete data; (2) conversion always loses information — exporting from RVT to IFC to OBJ progressively strips semantic meaning; (3) the most impactful AI applications target the gap between high-prevalence, low-machine-readability formats and the structured inputs that ML algorithms require.

| # | Format | Extension(s) | Structure Type | Machine/AI Readability | Typical Generation Context | Prevalence |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | IFC | .ifc, .ifcxml | Semi-structured | Medium-High | BIM authoring tools; open exchange standard | Very High |
| 2 | PDF | .pdf | Unstructured | Low | Specifications, contracts, reports, manuals | Very High |
| 3 | CSV | .csv, .tsv | Structured | High | Cost databases, sensor exports, schedule data | Very High |
| 4 | Excel | .xlsx, .xls | Structured | Medium | Cost plans, BOQs, COBie sheets, risk registers | Very High |
| 5 | JSON | .json | Semi-structured | High | IoT payloads, API responses, telematics feeds | High |
| 6 | XML | .xml | Semi-structured | High | Energy simulation (gbXML), specifications, regulatory submissions | High |
| 7 | SQL Databases | .sql, .db | Structured | High | CMMS/CAFM, ERP, cost control, payroll | High |
| 8 | JPEG/PNG | .jpg, .png | Unstructured | Medium (CV) | Site photographs, drone images, inspections | Very High |
| 9 | MP4/Video | .mp4, .avi | Unstructured | Medium (CV) | CCTV, drone video, time-lapse documentation | High |
| 10 | LAS/LAZ | .las, .laz | Semi-structured | Medium | Drone surveys, terrestrial laser scanning | High |
| 11 | COBie | .xlsx, .xml, .json | Structured/Semi | Medium-High | BIM extraction for handover to facility owners | High |
| 12 | BCF | .bcfzip | Semi-structured | Medium | Clash detection and coordination issue tracking | Medium-High |
| 13 | GeoJSON | .geojson | Semi-structured | High | GIS platforms; site boundaries, utility corridors | Medium-High |
| 14 | Shapefile | .shp (+sidecars) | Semi-structured | Medium | Government land registries, environmental layers | Medium-High |
| 15 | GeoTIFF | .tif | Binary/Encoded | Medium | Drone orthomosaics, satellite imagery, DEMs | Medium-High |
| 16 | RVT (Revit) | .rvt | Proprietary | Low | Revit BIM authoring (most widely used globally) | Very High |
| 17 | DWG/DXF | .dwg, .dxf | Proprietary/Semi | Low-Medium | 2D drawings, as-built documentation | Very High |
| 18 | XER (P6) | .xer | Structured | Medium | Primavera P6 schedule exchange | High |
| 19 | MPP | .mpp | Proprietary | Low | Microsoft Project scheduling | High |
| 20 | gbXML | .xml | Semi-structured | Medium-High | BIM-to-energy simulation interoperability | Medium |
| 21 | E57 | .e57 | Semi-structured | Medium | Terrestrial laser scanners, indoor scanning | Medium |
| 22 | MQTT | N/A (protocol) | Semi-structured | Medium-High | IoT sensor networks, BMS communication | Medium-High |
| 23 | BACnet/Modbus | N/A (protocols) | Structured | Medium | Building automation (HVAC, lighting, fire) | High (ops) |
| 24 | OPC-UA | N/A (protocol) | Structured | High | Industrial IoT, digital twin connectivity | Medium |
| 25 | DOCX | .docx | Unstructured | Low | Specifications, contracts, meeting minutes | Very High |
| 26 | Email | .msg, .eml | Unstructured | Low | RFIs, submittals, claims correspondence | Very High |
| 27 | KML/KMZ | .kml, .kmz | Semi-structured | Medium | Site boundary visualisation, stakeholder maps | Medium |
| 28 | OBJ/GLB/glTF | .obj, .glb | Semi-structured | Medium | VR/AR, web BIM viewers, digital twin visualisation | Medium |
| 29 | CityGML | .gml | Semi-structured | Medium | Urban planning, city-scale digital twins | Low-Medium |
| 30 | Parquet/Arrow | .parquet | Structured | Very High | Analytics-ready exports, ML feature stores | Low-Medium |
| 31 | DTDL | .json | Semi-structured | High | Azure Digital Twins platform configuration | Low |
| 32 | EDI | Various | Structured | Medium | Supply chain procurement, automated POs | Low-Medium |